# ARTIFACT

Модульная интерактивная party-машина

Software Development Plan

Version 1.0 — December 2025

VNVNC Nightclub, St. Petersburg

# Executive Summary

This document outlines the complete software development plan for ARTIFACT — a modular interactive party machine featuring a multi-layer display system (P3 LED 128×128px, WS2812B ticker 48×8px, LCD1601 16 chars), multiple input methods (USB big button, 3×4 keypad, arcade buttons, camera), thermal receipt printer, and AI-powered personalized experiences.

Target Launch: Polar Express event, December 26-27, 2025

Platform: Raspberry Pi 4 (8GB) running Raspberry Pi OS

Primary Language: Python 3.11+

# System Architecture Overview

The software architecture follows a modular, event-driven design with independent hardware abstraction layers for each component. A central state machine orchestrates mode transitions and synchronizes all displays and outputs.

## Core Architecture Diagram

┌─────────────────────────────────────────────────────────────────┐

│                    ARTIFACT MAIN CONTROLLER                     │

│  ┌──────────┐  ┌──────────┐  ┌──────────┐  ┌──────────────┐   │

│  │  STATE   │  │  EVENT   │  │   MODE   │  │    CONFIG    │   │

│  │ MACHINE  │◄─┤  QUEUE   │◄─┤ MANAGER  │  │   LOADER     │   │

│  └────┬─────┘  └──────────┘  └──────────┘  └──────────────┘   │

└───────┼─────────────────────────────────────────────────────────┘

│

▼

┌───────────────────────────────────────────────────────────────────┐

│                    HARDWARE ABSTRACTION LAYER                     │

├────────────┬────────────┬────────────┬────────────┬──────────────┤

│  P3 LED    │  WS2812B   │  LCD1601   │  PRINTER   │    SOUND     │

│  DRIVER    │  DRIVER    │  DRIVER    │  DRIVER    │   MANAGER    │

├────────────┼────────────┼────────────┼────────────┼──────────────┤

│  KEYPAD    │  ARCADE    │  USB BTN   │  CAMERA    │    GPIO      │

└────────────┴────────────┴────────────┴────────────┴──────────────┘

## Hardware Component Summary

| Component | Interface | Library | GPIO/Port |
| --- | --- | --- | --- |
| P3 LED 128×128 | HUB75 via HAT | rpi-rgb-led-matrix | RGB Matrix HAT |
| WS2812B Ticker | PWM | rpi_ws281x | GPIO 18 |
| LCD1601 I²C | I²C | RPLCD + smbus2 | GPIO 2, 3 |
| Thermal Printer | UART (TTL) | python-escpos | GPIO 14, 15 |
| Keypad 3×4 | Matrix GPIO | gpiozero / custom | GPIO 5,6,13,19,12,16,20 |
| USB Big Button | USB HID | evdev / keyboard | USB Port |
| Arcade Buttons ×2 | GPIO + Pull-up | gpiozero.Button | 2× GPIO (TBD) |
| Camera NoIR 12MP | CSI | picamera2 | CSI Port |
| USB Speakers | USB Audio | pygame.mixer | USB + 3.5mm jack |

# Development Phases

## Phase 0: Environment Setup (Dec 14-15)

Duration: 2 days  |  Priority: CRITICAL

- Install Raspberry Pi OS (64-bit Bookworm) on microSD

- Configure SSH, VNC, static IP for remote development

- Enable I²C, SPI interfaces via raspi-config

- Disable serial console for UART printer communication

- Disable onboard audio (conflicts with WS2812B PWM)

- Create Python virtual environment for the project

- Install all required Python packages and system dependencies

- Set up Git repository for version control

- Create project directory structure (see below)

### Project Directory Structure

artifact/

├── main.py                  # Application entry point

├── config/

│   ├── settings.yaml        # Global configuration

│   ├── gpio_pins.yaml       # GPIO pin assignments

│   └── themes/              # Event theme configs

├── drivers/

│   ├── led_matrix.py        # P3 HUB75 driver

│   ├── ticker.py            # WS2812B ticker driver

│   ├── lcd_display.py       # LCD1601 I2C driver

│   ├── printer.py           # Thermal printer driver

│   ├── camera.py            # Pi Camera driver

│   ├── keypad.py            # 3×4 matrix keypad

│   ├── buttons.py           # Arcade buttons handler

│   └── audio.py             # Sound manager

├── core/

│   ├── state_machine.py     # Main state controller

│   ├── event_bus.py         # Event queue system

│   ├── mode_manager.py      # Mode loading/switching

│   └── display_sync.py      # Multi-display sync

├── modes/

│   ├── base_mode.py         # Abstract mode class

│   ├── fortune_teller.py    # Прорицатель mode

│   ├── zodiac.py            # Зодиак mode

│   ├── roulette.py          # Рулетка mode

│   ├── quiz.py              # Викторина mode

│   ├── compatibility.py     # Совместимость mode

│   ├── generator.py         # Генератор mode

│   ├── ai_prophet.py        # AI-Пророк mode

│   └── lottery.py           # Лотерея mode

├── animations/

│   ├── matrix_anim.py       # P3 LED animations

│   ├── ticker_effects.py    # Ticker scroll effects

│   └── led_strip.py         # Ambient LED effects

├── assets/

│   ├── sounds/              # Sound effects (WAV/MP3)

│   ├── images/              # Animation frames (PNG)

│   ├── fonts/               # Pixel fonts for displays

│   └── data/                # Predictions, questions DB

├── ai/

│   ├── vision_analyzer.py   # Image analysis for AI mode

│   └── prediction_gen.py    # AI text generation

├── utils/

│   ├── cyrillic.py          # Cyrillic text handling

│   ├── image_utils.py       # Image processing helpers

│   └── logging_config.py    # Logging setup

## Phase 1: Hardware Drivers (Dec 16-19)

Duration: 4 days  |  Priority: CRITICAL

- Build and install rpi-rgb-led-matrix library (C++ with Python bindings)

- Create P3 LED driver class with 2×2 panel chaining (128×128 pixels)

- Configure RGB Matrix HAT with Address E line for 64×64 panels

- Install rpi_ws281x and create ticker driver for 48×8 pixel array

- Set up LCD1601 I²C with RPLCD library (16×1 character display)

- Configure UART and test python-escpos with EM5820 thermal printer

- Implement matrix keypad scanner with debouncing

- Set up arcade button handlers with gpiozero

- Configure USB HID listener for big red button (keyboard emulation)

- Initialize pygame.mixer for sound output via USB speakers

## Phase 2: Core Framework (Dec 19-21)

Duration: 3 days  |  Priority: HIGH

- Implement event bus with asyncio for non-blocking event handling

- Build state machine: IDLE → INPUT → PROCESSING → RESULT → PRINT

- Create display synchronization layer (all 3 displays update together)

- Develop abstract BaseMode class defining mode interface

- Implement configuration loader (YAML) for themes and settings

- Add graceful shutdown handling (cleanup GPIO, stop threads)

- Create systemd service for auto-start on boot

## Phase 3: Animation Engine (Dec 20-22)

Duration: 3 days  |  Priority: HIGH

- Implement frame-based animation system for P3 matrix (target: 30fps)

- Create Cyrillic pixel font renderer (5×7 or 8×8 fonts)

- Build ticker scrolling text with variable speed support

- Develop effect library: fade, wipe, spiral, typewriter, particles

- Create idle animation loop (mystical symbols, particles, etc.)

- Optimize rendering with double-buffering for flicker-free display

## Phase 4: Mode Implementation (Dec 21-24)

Duration: 4 days  |  Priority: HIGH

- FORTUNE TELLER (Прорицатель): Random prediction from database, one-time display per night

- ZODIAC (Зодиак): Date input → zodiac detection → horoscope display

- ROULETTE (Рулетка): Wheel spin animation with configurable sectors

- QUIZ (Викторина): Questions with timer, score tracking, 4-choice answers

- COMPATIBILITY (Совместимость): Two-player input, percentage calculation

- GENERATOR (Генератор): Slot-machine style random combinations

- LOTTERY (Лотерея): 3-number selection, random draw, match display

- Create receipt templates for thermal printer output

## Phase 5: Integration Testing (Dec 24-25)

Duration: 2 days  |  Priority: CRITICAL

- Full hardware integration test (all components together)

- Stress test: continuous operation for 6+ hours

- User flow testing: complete interaction cycles

- Error recovery testing: power loss, disconnected components

- Performance profiling and optimization

- Bug fixes and stability improvements

## Phase 6: AI-Prophet Mode (Post-Dec 27)

Duration: Ongoing  |  Priority: MEDIUM (after launch)

- Install picamera2 and test Camera Module 3 NoIR

- Implement face detection and image capture workflow

- Integrate Claude API for personalized prediction generation

- Build image analysis prompt templates

- Create countdown and flash animation for photo capture

- Target: Ready for Raven Ball (Dec 31)

# Dependencies and Installation

## System Dependencies (apt)

# System packages

sudo apt update && sudo apt install -y \

python3-pip python3-venv python3-dev \

git build-essential libpng-dev libjpeg-dev \

python3-smbus i2c-tools \

libsdl2-mixer-2.0-0 libsdl2-image-2.0-0 \

python3-picamera2 libcamera-dev

## Python Dependencies (pip)

# requirements.txt

rpi_ws281x>=5.0.0        # WS2812B LED control

adafruit-circuitpython-neopixel>=6.3.0

RPLCD>=1.4.0             # LCD control

smbus2>=0.4.0            # I2C communication

python-escpos>=3.0       # Thermal printer

gpiozero>=2.0            # GPIO control

evdev>=1.6.0             # USB HID devices

pygame>=2.5.0            # Sound playback

Pillow>=10.0.0           # Image processing

numpy>=1.24.0            # Array operations

pyyaml>=6.0              # Configuration

anthropic>=0.20.0        # Claude API (for AI mode)

## rpi-rgb-led-matrix Installation

# Clone and build from source (required for HUB75)

git clone https://github.com/hzeller/rpi-rgb-led-matrix.git

cd rpi-rgb-led-matrix

make build-python PYTHON=$(which python3)

sudo make install-python PYTHON=$(which python3)

## System Configuration

# /boot/config.txt additions

dtparam=audio=off        # Disable audio (WS2812B needs PWM)

dtparam=i2c_arm=on       # Enable I2C

dtparam=spi=on           # Enable SPI

enable_uart=1            # Enable UART for printer

dtoverlay=disable-bt     # Free up UART pins

# Timeline Summary

| Date | Phase | Deliverables | Milestone |
| --- | --- | --- | --- |
| Dec 14-15 | Phase 0 | Dev environment ready | Pi boots, SSH works |
| Dec 16-19 | Phase 1 | All drivers functional | Hardware verified |
| Dec 19-21 | Phase 2 | Core framework done | State machine works |
| Dec 20-22 | Phase 3 | Animation engine | Visuals running |
| Dec 21-24 | Phase 4 | All modes implemented | Feature complete |
| Dec 24-25 | Phase 5 | Full integration testing | Ready for install |
| Dec 26-27 | LAUNCH | POLAR EXPRESS | FIRST PUBLIC TEST |
| Post Dec 27 | Phase 6 | AI-Prophet mode | Ready for Raven Ball |

# Risks and Mitigations

- USB Button Delivery (arrives Dec 25): Mitigation: Implement keyboard emulation for Enter key during development. Can test all flows without physical button.

- P3 Panel Availability (Avito): Mitigation: Have backup seller identified. Confirm pickup date Dec 16-17. Start driver dev with simulation.

- WS2812B + Audio Conflict: Mitigation: Use USB audio output instead of onboard audio. WS2812B uses GPIO 18 PWM which conflicts with audio.

- Power Supply Stability: Mitigation: Test full brightness on all LEDs + printer simultaneously. Mean Well LRS-200-5 provides 40A headroom.

- Cyrillic Display Support: Mitigation: Create custom pixel fonts early. LCD1601 may need custom character generation. Thermal printer supports CP866.

- Time Pressure: Mitigation: Prioritize Fortune Teller + Roulette for Dec 26. Other modes can be added incrementally.

— End of Document —

Generated: December 2025 | VNVNC Internal Use Only